# Scope of Work

## Refrigeration Pipe Equivalent Length, Fittings & Line Sizing Calculator

### 1. Objective

Develop a web-based Refrigeration Piping Design & Line Sizing Calculator in HTML/CSS/JavaScript that combines:

- Refrigerant line sizing

- Actual pipe length calculation

- Equivalent length of pipe fittings and components

- Pressure-drop calculation

- Refrigerant velocity calculation

- Oil-return assessment

- Engineering PASS / WARNING / FAIL indicators

- Multi-unit input and output

- Engineering guidance to support refrigeration design decisions

The tool should provide functionality comparable to the Sporlan Virtual Engineer, while adding a dedicated and more flexible actual equivalent-length calculation module, which is not directly provided by Sporlan Virtual Engineer.

# 2. Main Calculation Modules

### A. Refrigeration System Inputs

The user shall be able to define:

- Refrigerant

- Refrigeration capacity

- Evaporating temperature / pressure

- Condensing temperature / pressure

- Suction temperature / superheat

- Liquid temperature / subcooling

- Refrigerant mass flow

- Pipe material

- Pipe type

- Pipe nominal size

- Actual pipe length

- Vertical rise / drop

- Number and type of fittings

- Valves

- Filters/driers

- Sight glasses

- Solenoid valves

- Check valves

- Other refrigeration devices

The system should support common refrigerants, including:

- R454B

- R410A

- R32

- R134a

- R448A

- R449A

- R407C

- R404A

- R1234yf

- R1234ze

- R22

- R507A

- R744

- R717

- R290

Additional refrigerants should be expandable through the database. OR use REFPROP or Cool prop

# 3. Multi-Unit Input and Output

All major inputs shall support multiple engineering units.

Examples:

Length

- ft

- in

- m

- mm

Diameter

- in

- mm

Pressure

- psig

- psia

- bar

- kPa

- MPa

Temperature

- °F

- °C

- K

Capacity

- Btu/h

- MBH

- ton

- kW

Mass Flow

- lb/h

- lb/min

- kg/h

- kg/s

Velocity

- ft/min

- ft/s

- m/s

Pressure Drop

- psi

- psi/100 ft

- kPa

- bar

- K

The user may select the preferred unit system independently for input and output where practical.

# 4. Actual Equivalent Length Calculation

The key enhancement over existing online line-sizing tools shall be the ability to calculate actual equivalent piping length from the user’s piping configuration.

The user shall enter:

### Straight Pipe

Example:

| Pipe Section | Length |
| --- | --- |
| Horizontal | 80 ft |
| Vertical Rise | 20 ft |
| Horizontal | 50 ft |
| Vertical Drop | 10 ft |

The system calculates the total actual pipe length.

### Fittings

The user shall be able to add multiple fittings:

- 90° elbows

- 45° elbows

- Long-radius elbows

- Short-radius elbows

- Tees

- Reducers

- Expansions

- Couplings

- Branch connections

- U-bends

- Return bends

- Other applicable fittings

Example:

| Component | Quantity | Equivalent Length/Component | Total |
| --- | --- | --- | --- |
| 90° Elbow | 12 | 3.5 ft | 42 ft |
| 45° Elbow | 4 | 1.5 ft | 6 ft |
| Tee | 2 | 8 ft | 16 ft |

### Refrigeration Components

The database should also include equivalent-length or pressure-drop information for:

- Ball valves

- Globe valves

- Solenoid valves

- Check valves

- Filter-driers

- Sight glasses

- Service valves

- Pressure regulators

- Expansion devices

- Distributors

- Receivers

- Accumulators

- Oil separators

- Other commonly used refrigeration components

# 5. Equivalent Length Calculation

The tool shall calculate:

Total Equivalent Length = Actual Straight Pipe Length + Equivalent Length of Fittings + Equivalent Length of Components

The calculation shall separately display:

- Actual pipe length

- Fitting equivalent length

- Valve/component equivalent length

- Vertical piping

- Total equivalent length

- Equivalent-length multiplier

The user should be able to see exactly how the final equivalent length was obtained.

# 6. Fitting Database

The attached Excel spreadsheet shall be used as the initial database for:

- Fitting types

- Valve types

- Refrigeration devices

- Equivalent lengths

- Pipe sizes

- Associated calculation values

The database shall then be reviewed and expanded using globally recognized refrigeration and piping references/standards.

Potential reference sources should include applicable requirements and engineering guidance from organizations such as:

- ASHRAE

- ASME

- AHRI

- IIAR

- CSA

- ISO

- EN standards

- ASHRAE Handbook – Refrigeration

- Manufacturer engineering data

- Refrigeration piping design manuals

Manufacturer-specific data should be identified separately from standardized engineering data.

# 7. Refrigerant Line Sizing

The tool shall calculate and/or evaluate:

### Suction Line

- Recommended pipe diameter

- Refrigerant velocity

- Pressure drop

- Equivalent length

- Saturated suction temperature loss

- Oil-return capability

- Vertical-riser performance

- Minimum velocity requirement

### Discharge / Hot-Gas Line

- Recommended pipe diameter

- Refrigerant velocity

- Pressure drop

- Equivalent length

- Condensing-temperature impact

- Oil-return capability

### Liquid Line

- Recommended pipe diameter

- Refrigerant velocity

- Pressure drop

- Flash-gas risk

- Available subcooling

- Static head effect

- Equivalent length

Where applicable, the calculation shall account for:

- Horizontal piping

- Vertical risers

- Vertical drops

- Part-load operation

- Reduced mass flow

- Multiple circuits

- Compressor staging

- VFD operation

- One circuit operating while another is off

# 8. Pressure-Drop Analysis

The tool shall calculate:

- Total pressure drop

- Pipe friction pressure drop

- Fitting pressure drop

- Component pressure drop

- Static pressure/head effect where applicable

- Total equivalent pressure loss

- Pressure drop per 100 ft

- Equivalent saturated-temperature drop

The user shall be able to see the contribution from each component.

# 9. Refrigerant Velocity Analysis

The tool shall calculate refrigerant velocity through the selected pipe.

Results should include:

- ft/min

- ft/s

- m/s

Velocity shall be evaluated separately for:

- Horizontal suction line

- Vertical suction riser

- Discharge line

- Liquid line

The system shall provide engineering guidance regarding whether the calculated velocity is:

🟢 Acceptable

🟡 Marginal / Review

🔴 Not Recommended

The limits should not be hard-coded blindly. They should be associated with the refrigerant, line type, operating condition, and engineering reference used.

# 10. Oil Return Assessment

For suction and discharge piping, the tool should evaluate the likelihood of adequate oil return.

The calculation should consider:

- Refrigerant velocity

- Pipe diameter

- Mass flow

- Vertical riser

- Refrigerant density

- Oil-return velocity requirements

- Part-load operation

- Compressor staging

- VFD turndown

The tool should identify situations such as:

WARNING – Velocity may be insufficient for reliable oil return during minimum-load operation.

This is particularly important for systems where the compressor operates at reduced capacity or where only one refrigeration circuit operates at a time.

# 11. Engineering Decision Dashboard

The calculator should provide an engineering dashboard rather than only numerical results.

Example:

| Parameter | Result | Status | Engineering Interpretation |
| --- | --- | --- | --- |
| Pipe Size | 1-3/8 in | 🟢 | Suitable |
| Equivalent Length | 165 ft | 🟢 | Within design range |
| Suction Velocity | 720 ft/min | 🟢 | Suitable for oil return |
| Pressure Drop | 2.1 psi | 🟢 | Acceptable |
| SST Drop | 1.2°F | 🟢 | Acceptable |
| Minimum-Load Velocity | 310 ft/min | 🟡 | Review oil return |
| Liquid Pressure Drop | 8.2 psi | 🟡 | Check available subcooling |

# 12. Engineering Guidance / Information Icons

Every major result should have an ⓘ Information icon.

When selected, it should explain:

- What the parameter means

- Why it matters

- Typical acceptable range

- What happens if the value is too high

- What happens if the value is too low

- Applicable engineering reference

- Whether the value is a design recommendation, manufacturer limit, or standard requirement

Example:

ⓘ Suction Velocity

Higher velocity generally improves oil entrainment and return through vertical risers, but excessive velocity can increase pressure drop, noise, and operating losses. Low velocity may create oil-return concerns, particularly during low-load operation.

The tool should clearly distinguish between:

Code requirement → Standard requirement → Manufacturer requirement → Engineering recommendation

These should not be presented as interchangeable.

# 13. Standards & Reference Framework

The calculation engine should be developed using recognized refrigeration and piping engineering references.

The application should maintain a reference database containing:

- Standard/reference name

- Edition/year

- Applicable refrigerants

- Applicable line type

- Parameter

- Recommended range

- Source/reference

- Notes and limitations

The application should avoid presenting a generic velocity or pressure-drop value as an absolute requirement when the applicable standard provides only guidance.

# 14. Pipe Size Selection

The user should be able to:

### Option 1 – Manual Selection

Select:

Refrigerant → Line Type → Pipe Material → Pipe Size

The tool evaluates the selected pipe.

### Option 2 – Automatic Selection

The tool evaluates available pipe sizes and recommends:

Recommended Pipe Size: 1-3/8 in

It should also show:

- Smaller size

- Recommended size

- Larger size

and compare:

- Velocity

- Pressure drop

- Equivalent length

- Oil return

- Temperature drop

This allows the engineer to make an informed final selection.

# 15. Calculation Transparency

The application shall not function as a “black box.”

For every major calculation, the user should be able to expand:

Show Calculation

and see:

- Formula

- Input values

- Unit conversion

- Intermediate calculations

- Final result

- Reference/source

Example:

Total Equivalent Length

Leq = Lpipe + Σ(Lfitting × Quantity) + Σ(Lcomponent × Quantity)

The application should show the actual numbers used in the calculation.

# 16. Piping Configuration Builder

Provide an interactive table where the user can build the actual piping system.

Example:

| # | Component | Size | Qty | Actual Length | Equivalent Length | Total |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Straight Pipe | 1-3/8” | 1 | 80 ft | — | 80 ft |
| 2 | 90° Elbow | 1-3/8” | 8 | — | 3.5 ft | 28 ft |
| 3 | Ball Valve | 1-3/8” | 1 | — | 5 ft | 5 ft |
| 4 | Filter-Drier | 1-3/8” | 1 | — | 8 ft | 8 ft |

Total Equivalent Length = 121 ft

Rows should be dynamically added or removed.

# 17. Results and Reporting

The application should provide:

- Summary dashboard

- Detailed calculation sheet

- Pipe-sizing comparison

- Equivalent-length breakdown

- Pressure-drop breakdown

- Velocity analysis

- Oil-return assessment

- Engineering warnings

- References

The user should be able to export the results as:

- PDF

- Excel

- CSV

- Printable engineering report

# 18. Data Quality & Traceability

Every fitting/component value shall have a source.

The database should identify whether a value comes from:

- Standard

- Manufacturer data

- Engineering reference

- Published fitting-loss data

- User-defined value

User-defined values should be supported where manufacturer-specific data is available.

The application should allow future updates to the fitting/component database without changing the calculation engine.

# 19. User Interface

The interface should be clean and engineering-focused.

Recommended workflow:

1. System Definition
↓
2. Refrigerant & Operating Conditions
↓
3. Pipe Configuration
↓
4. Fittings & Components
↓
5. Line Size Selection
↓
6. Equivalent Length Calculation
↓
7. Pressure Drop & Velocity
↓
8. Oil Return Assessment
↓
9. Engineering Decision Dashboard
↓
10. Report

The interface should use clear status indicators:

🟢 PASS / ACCEPTABLE
🟡 WARNING / ENGINEERING REVIEW
🔴 FAIL / NOT RECOMMENDED

# 20. Primary Goal

The final application should go beyond a conventional refrigeration line-sizing calculator.

It should answer the engineer’s practical questions:

What pipe size should I use?

What is the actual equivalent length of my piping system?

What is the pressure drop?

Is my refrigerant velocity acceptable?

Will I have adequate oil return?

What happens at minimum load?

Is my line size too small or unnecessarily large?

What engineering standard or reference supports this decision?

The final result should therefore function as an engineering decision-support tool for refrigeration piping design, combining transparent calculations, actual piping configuration, equivalent-length analysis, refrigerant line sizing, and standards-based engineering guidance.